LOYOLA ACADEMY DEGREE & PG COLLEGE

OLD ALWAL, SECUNDERABAD - 500 010, TELANGANA, INDIA

An Autonomous Institution Affiliated to Osmania University

Re-accredited with ‘A’ Grade (III Cycle) by NAAC A “College with Potential for Excellence” by UGC

Practical Record

CERTIFICATE

This is to certify that this is a Bonafide record work done in ----------------------- practical during - ---year	semester of the academic year 202 —202-

Name:

UID No:

Class:

Signature of Internal	Signature of HoD

Signature of External	Signature of Principal

INDEX

| S.No | Experiment Name | Date | Signature |
| --- | --- | --- | --- |
| 1. | Configure OSPF MD5 Authentication |  |  |
| 2. | Configure NTP server |  |  |
| 3. | Configure Routers to Log Messages to the Syslog Server |  |  |
| 4. | Configure Router to Support SSH Connections |  |  |
| 5. | Configure server-based AAA authentication |  |  |
| 6. | Configure server-based AAA authentication using TACACS+ |  |  |
| 7. | Configuring Server-Based AAA Authentication Using RADIUS |  |  |
| 8. | Configuring ACLs |  |  |

# Program No:01	                                                                                                   Date:

# Configure OSPF MD5 Authentication

- Open the cisco packet tracer and place 3 routers and rename the display name as R1, R2, R3 and the hostname as the same R1, R2, R3 (In the config tab).

- Then go to R1 – physical tab- switch off the router- place the module name HWIC-2T in the port- switch the router.

- Repeat the same for all the 3 routers.

- Then we need to have the connections between the routers- we need to have serial DTE connection.

have the connection from R1-R2 (use serial0/1/0) and for R2-R3(use serial0/1/1)

- Then have SWITCHES, SERVERS (SYSLOG SERVER & NTP SERVER) and a PC

(SSH CLIENT) in the network and use the auto connectors for connecting the devices.

Build the network as shown in the figure.

- Now configure the network-

  - Let us have 192.168.1.0 for the first area.

  - Let us have 10.1.1.0 for the second area.

  - Let us have 10.2.2.0 for the third area.

  - Let us have 192.168.3.0 for the fourth area.

- Check the interface at router 1 to switch0 and check the port gig0/0 and configure the Ip address as 192.168.1.1 and subnet mask as 255.255.255.0 and click on.

- Go to the NTP server and go to the desktop and set the Ip address as 192.168.1.2 and subnet mask as 255.255.255.0 and the default gateway will be 192.168.1.1.

- Same for SYSLOG server, go to the desktop- set the Ip address as 192.168.1.3 and the subnet mask as 255.255.255.0 and the default gateway as 192.168.1.1.

- Now configure routers

  - R1- Go to serial0/1/0 and add the Ip address as 10.1.1.1 and subnet mask as 255.255.255.0 and switch it on.

  - R2- Go to the serial0/1/0 and the Ip address as 10.1.1.2 and the subnet mask as 255.255.255.0 and switch it on.

  - R2- Go to the serial0/1/1 and the Ip address as 10.2.2.1 and subnet mask as 255.255.255.0 and switch it on.

  - R3- Go to the serial0/1/1 and the Ip address as 10.2.2.2 and subnet mask as 255.255.255.0 and switch it on.

- Now configure the R3-pc area

  - Go to the R3 router and click on connection gig0/1 add the Ip address as 192.168.3.1 and the subnet mask as 255.255.255.0 and switch it ON.

  - The go to PC click on the desktop and add the Ip address as 192.168.3.2 and the subnet mask as 255.255.255.0 and the default gateway as 192.168.3.1.

CONFIGURING OSPF ROUTING-

- Go to R1 - Go to CLI tab

  - Router>en

  - Router#conf t

  - Router(config)#router ospf 1

  - Router(config-router)#network 192.168.1.0  0 .0.0.255 area 1

  - Router(config-router)#network 10.1.1.0  0.0.0.255 area 0

  - Router(config-router)#end

  - Router#

- Go to R2 - Go to CLI tab

  - Router>en

  - Router#conf t

  - Router(config)#router ospf 1

  - Router(config-router)#network 10.1.1.0 0 .0.0.255 area 0

  - Router(config-router)#network 10.1.1.0  0.0.0.255 area 0

  - Router(config-router)#network 10.2.2.0  0.0.0.255 area 0

  - Router(config-router)#end

  - Router#

- Go to R3 - Go to CLI tab

  - Router>en

  - Router#conf t

  - Router(config)#router ospf 1

  - Router(config-router)#network 10.2.2.0  0 .0.0.255 area 0

  - Router(config-router)#network 192.168.3.0  0.0.0.255 area 2

  - Router(config-router)#end

  - Router#

CONFIGURE OSPF MD5 AUTHENTICATION.

R1- Go to the CLI tab

- Router>en

- Router#conf t

- Router(config)#router ospf 1

- Router(config-router)# area 0 authentication message-digest

- Router(config-router)# exit

- Router(config)# int se0/1/0

- Router(config-if)# ip ospf message-digest-key 1 md5 MD5pa55

- Router(config-if)#

R1- Go to the CLI tab

- Router>en

- Router#conf t

- Router(config)#router ospf 1

- Router(config-router)# area 0 authentication message-digest

- Router(config-router)# exit

- Router(config)# int se0/1/0

- Router(config-if)# ip ospf message-digest-key 1 md5 MD5pa55

- Router(config-if)#exit

- Router(config)# int se0/1/1

- Router(config-if)# ip ospf message-digest-key 1 md5 MD5pa55

R3- Go to the CLI tab

- Router>en

- Router#conf t

- Router(config)#router ospf 1

- Router(config-router)# area 0 authentication message-digest

- Router(config-router)# exit

- Router(config)# int se0/1/1

- Router(config-if)# ip ospf message-digest-key 1 md5 MD5pa55

- Router(config-if)#exit

- Router(config)#exit

- Router# show ip ospf

# Program No:02                                                                                                 Date:

# Configure NTP server

  - Open the cisco packet tracer and open the same network which we have used before.

  - Now enable the NTP server, so click on NTP server and go to services tab- go to NTP and click on enable.

  - Enter the key-1 and the password – NTPpa55 and switch it on.

  - Go to router R1 and go to CLI tab-

    - Router>en

    - Router#conf t

    - Router(config)#ntp server 192.168.1.2

    - Router(config)# exit

    - Router# show ntp status

    - Router# conf t

    - Router(config)#ntp update-calendar

    - Router(config)#ntp authenticate

    - Router(config)#ntp trusted-key 1

    - Router(config)#ntp authentication-key 1 md5 NTPpa55

    - Router(config)#exit

    - Router#show ntp status

  - Repeat the same for all the 3 routers.

# Program No:03	                                                                                                   Date:

# Configure Routers to Log Messages to the Syslog Server

  - Open the cisco packet tracer and open the same network which we have used before.

  - Click on R1 and go to CLI-

    - Router# conf t

    - Router(config)# logging host 192.168.1.3

    - Router(config)# exit

    - Router# show logging

  - Repeat the same for R2

    - Router>en

    - Router# conf t

    - Router(config)# logging host 192.168.1.3

    - Router(config)# exit

    - Router# show logging

  - Now go to the SYSLOG server and click on services- go to SYSLOG and we can see the message are generating.

# Program No:04                                                                                                 Date:

# Configure Router to Support SSH Connections

  - Open the cisco packet tracer and open the same network which we have used before.

  - Click on R3 and go to CLI-

    - Router>en

    - Router#conf

    - Router(config)#ip domain-name ccnasecurity.com

    - Router(config)#username SSHadmin privilege 15 secret ciscopa55

    - Router(config)#line vty 0 4

    - Router(config-line)#login local

    - Router(config-line)#transport input ssh

    - Router(config-line)#crypto key generate rsa Change the router name as R3

    - R3(config)#crypto key generate rsa How many bits in the modulus [512]:1024

    - R3(config)#exit

    - R3# show ip ssh

  - Now go to the PC and go to desktop- command prompt

  - C:\telnet 192.168.3.1

  - C:\>ssh -l SSHadmin 192.168.3.1 Password: ciscopa55

# Program No:05                                                                                                   Date:

# Configure server-based AAA authentication

  - Open cisco packet tracer and place 3 routers (1941), 3 switches (2960) , 3 PC and 2 servers and connect the devices as shown in the figure.

  - And have the serial connections between routers so add the port (HWIC-2T) to the routers and connect them with serial DTE connection.

  - Go to the router R1 and click on gig0/0 – ip address- 192.168.1.1 & subnet mask- 255.255.255.0 and switch it on.

  - Go to pc0- desktop- ip address- 192.168.1.2 & subnet mask- 255.255.255.0 and default gateway as 192.168.1.1

  - Now go to R2 and click on gig0/0- ip address-192.168.2.1 and subnet mask is 255.255.255.0 and switch it on.

  - Go to pc1- desktop- op address- 192.168.2.2 & subnet mask as 255.255.255.0 and default gateway as 192.168.2.1

  - Go to the server- desktop- ip address- 192.168.2.3 & subnet mask as 255.255.255.0 and default gateway as 192.168.2.1.

  - Now go to R3 and set the ip configuration in gig0/0- ip address-192.168.3.1 and the subnet mask as 255.255.255.0 and switch it on.

  - Now go to PC3 and set the ip config as 192.168.3.2 and subnet mask as 255.255.255.0 and default gateway will be 192.168.3.1

  - Go to server- desktop- IP address-192.168.3.3 and the subnet mask as 255.255.255.0 and the default gateway as 192.168.3.1

  - Now configure the routers:

    - (R1- R2)- serial0/1/0- ip address-10.1.1.1 (R1) & subnet mask- 255.255.255.0(R1)

    - (R1-R2)- serial0/1/0-ip address- 10.1.1.2(R2) & subnet mask- 255.255.255.0 (R2)

    - (R2-R3)- serial0/1/1- ip address- 10.2.2.1 (R2) & subnet mask- 255.255.255.0 (R2).

    - (R2-R3)- serial0/1/1- ip address- 10.2.2.2 (R3) & subnet mask- 255.255.255.0 (R3).

  - Add RIP to all the routers.

    - R1- 192.168.1.0 & 10.1.1.0

    - R2- 192.168.2.0, 10.1.1.0 & 10.2.2.0

    - R3- 192.168.3.0 & 10.2.2.0

Configure local AAA authentication for console access

  - Go to the router R1 – Go to the CLI

    - Router#conf t

    - Router(config)#username Admin1 secret admin1pa55

    - Router(config)#aaa new-model

    - Router(config)#aaa authentication login default local

    - Router(config)#line console 0

    - Router ( config-line)# login authentication default

    - Router (config-line)# end

    - Router#exit

USERNAME: Admin1

PASSWORD: admin1pa55

# Program No:06                                                                                    Date:

# Configure server-based AAA authentication using TACACS+

  - Go to the same network of AAA which we have configured before and rename the server as TACACS.

  - Click on router R2 and go to CLI tab

    - R2#conf t

    - R2(config)#username Admin2 secret admin2pa55

  - Now go to the TACACS sever and go to services tab- click on AAA Client-name – R2

Client ip- 192.168.2.1

Secret – 654321 Servertype- tacacs Now click on add

  - User setup- Username- Admin2

Password- admin2pa55

Now switch on the server.

  - Go to R2- CLI mode

    - R2(config)#tacacs-server host 192.168.2.3

    - R2(config)#tacacs-server key 654321

    - R2(config)#aaa new-model

    - R2(config)#aaa authentication login default group tacacs+ local

    - R2(config)#line console 0

    - R2(config-line)#login authentication default

    - R2(config-line)#end

    - R2(config)#exit

USERNAME: Admin2

PASSWORD: admin2pa55

# Program No:07                                                                                                Date:

# Configuring Server-Based AAA Authentication                                                                                       Using RADIUS

  - Go to the same network of AAA which we have configured before and rename the server as RADI US.

  - Go to R3 router- CLI tab

    - R3>

    - R3>en

    - R3# conf t

    - R3(config)#username Admin3 secret admin3pa55

  - Go to RADIUS server- go to services – click on AAA

  - Network configuration Client name- R3 Client ip- 192.168.3.1

Secret- 123456 Servertype- radius Now click on add

  - User setup Username-Admin3

Password- admin3pa55

And click on add and switch on the server

  - Go to R3 router- CLI tab

    - R3(config)#radius-server host 192.168.3.3

    - R3(config)#raduis-server key 123456

    - R3(config)#aaa new-model

    - R3(config)#aaa authentication login default group radius local

    - R3(config)#line console 0

    - R3(config-line)#login authentication default

    - R3(config-line)#end

    - R3#exit

USERNAME: Admin3

PASSWORD: admin3pa55

# Program No:08                                                                                                   Date:

# Configuring	ACLs

  - Open cisco packet tracer and place 3 routers (1941), 2 switches (2960) , 1 PC and 1 servers and connect the devices as shown in the figure

  - And have the serial connections between routers so add the port (HWIC-2T) to the routers and connect them with serial DTE connection.

  - Go to the router R1 and click on gig0/0 – ip address- 192.168.1.1 & subnet mask- 255.255.255.0 and switch it on.

  - Go to server (PC-A)- desktop- ip address- 192.168.1.2 & subnet mask- 255.255.255.0 and default gateway as 192.168.1.1

  - Now go to R3 and click on gig0/0- ip address-192.168.3.1 and subnet mask is 255.255.255.0 and switch it on.

  - Go to pc-C- desktop- op address- 192.168.3.2 & subnet mask as 255.255.255.0 and default gateway as 192.168.3.1

  - Now configure the routers:

    - (R1- R2)- serial0/1/0- ip address-10.1.1.1 (R1) & subnet mask- 255.255.255.0(R1)

    - (R1-R2)- serial0/1/0-ip address- 10.1.1.2(R2) & subnet mask- 255.255.255.0 (R2)

    - (R2-R3)- serial0/1/1- ip address- 10.2.2.1 (R2) & subnet mask- 255.255.255.0 (R2).

    - (R2-R3)- serial0/1/1- ip address- 10.2.2.2 (R3) & subnet mask- 255.255.255.0 (R3).

  - Now set the loopback Ip address- Go to the cli

    - Router(config)#interface loopback 1

    - Router(config-if)#ip address 192.168.2.1 255.255.255.0

    - Router(config)#exit

  - Add RIP to all the routers.

    - R1- 192.168.1.0 & 10.1.1.0

    - R2- 192.168.2.0, 10.1.1.0 & 10.2.2.0

    - R3- 192.168.3.0 & 10.2.2.0

  - Go to R1 – CLI

    - R1(config)#ip domain-name ccnasecurity.com

    - R1(config)#username Admin1 privilege 15 secret cisco1

    - R1(config)#line vty 0 4

    - R1(config-line)#login local

    - R1(config-line)#transport input ssh

    - R1(config-line)#exit

    - R1(config)#crypto key generate rsa how many bits in the module [512] : 1024

    - R1(config)#exit

    - R1# show ip ssh